CPE301 – SPRING 2019

# Design Assignment 2A

Student Name: Ron Joshua Recrio

Student #: 5003825419

Student Email: recrio@unlv.nevada.edu

Primary Github address: https://github.com/recrio/submissions

Directory: /DesignAssignments/DA2A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

ATmega328p Xplained Mini

Multifunction Shield

Block diagram with pins used in the Atmega328P

Task 1:

Task 2:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

.ORG 0x0000

init:

LDI R16, 4			; 0b000100 or PB2

OUT DDRB, R16			; PB2 set as an output

LDI R17, 0			; Used for toggling PB2

LDI R20, 5			; 0b101 for 1024 prescaler

STS TCCR1B, R20		; prescaler set

LDI R20, 0			; to be used for TCNT1 reset

main:

RCALL TCNT1_RESET		; reset TCNT1

RCALL delay_ON		; call for .435 second delay

EOR R17, R16			; XOR toggles the LED

OUT PORTB, R17		; should turn off the LED

RCALL TCNT1_RESET		; reset TCNT1

RCALL delay_OFF		; call for .290 second delay

EOR R17, R16			; toggle LED again

OUT PORTB, R17		; should turn on the LED

RJMP main			; repeat

TCNT1_RESET:

STS TCNT1H, R20		; timer/counter

STS TCNT1L, R20		; reset

RET

check_high_ON:

CPI R27, 0x1A			; check if high byte of timer matches 0x1A

BRLT delay_ON			; if not, recheck timer

RET				; return to main

delay_OFF:				; TCNT should be 4531 or 0x11B3

LDS R27, TCNT1H		; R27 = high byte of timer/counter

LDS R26, TCNT1L		; R26 = low byte of timer/counter

CPI R26, 0xB3			; check if low byte of timer matches 0x8C

BRSH check_high_OFF		; if low bytes match, check high byte next

RJMP delay_OFF		; repeat until matching

check_high_OFF:

CPI R27, 0x11			; check if high byte of timer matches 0x1A

BRLT delay_OFF		; if not, recheck timer

RET				; return to main

delay_ON:				; TCNT should be 6796 or 0x1A8C

LDS R27, TCNT1H		; R27 = high byte of timer/counter

LDS R26, TCNT1L		; R26 = low byte of timer/counter

CPI R26, 0x8C			; check if low byte of timer matches 0x8C

BRSH check_high_ON		; if low bytes match, check high byte next

RJMP delay_ON			; repeat until matching

- VERIFICATION OF TASK 1 USING C

#include <stdio.h>

#include <avr/io.h>

int main(void)

{

DDRB = (1<<2); //Make PB2 Output

TCCR1B = 5;	// set prescaler to 1024

while (1)

{

TCNT1 = 0; // set timer/counter to 0

while (TCNT1 != 6796) {

//do nothing

}

PORTB ^= (1<<2); // toggle PB2 using xor

TCNT1 = 0; // reset again

while (TCNT1 != 4531) {

// do nothing

}

PORTB ^= (1<<2); // toggle PB2 using xor

}

}

- DEVELOPED CODE OF TASK 2/A

.ORG 0

init:

CBI DDRC, 2		; PC2 as input, Clears DDRC2

LDI R16, 4		; PB2

OUT DDRB, R16	; SET PB2 as output

LDI R17,0		; toggle for PC2

LDI R20,5		; 0b101 for prescaler 1024

STS TCCR1B, R20	; Set prescaler to 1024

LDI R20, 0		; For counter/timer reset

EOR R17, R16		; Toggle with XOR so that it starts as OFF

main:

OUT PORTB, R17	; Turn off LED

SBIC PINC, 2		; Skip if PC2 is pressed

RJMP keep_checking	; if not then jump

EOR R17, R16		; Toggle again

OUT PORTB, R17	; Turn on LED

EOR R17, R16		; Toggle back

STS TCNT1H, R20	; Reset

STS TCNT1L, R20	; Timer/Counter

RCALL delay		; call delay of 1.25s

RJMP main		; Loop again

keep_checking:

SBI PORTB, 2		; PB2 = 1, set it high so it will not light up

RJMP main		; Loop back to check for button press

delay:			; TCNT1 should be 19,531 or 0x4C4B

LDS R27, TCNT1H	; high byte of timer/counter

LDS R26, TCNT1L	; low byte of timer/counter

CPI R26,0x4B		; check if low byte matches

BRSH check_high	; if matching, then check high byte

RJMP delay		; else keep checking

check_high:

CPI R27,0x4C		; check high byte

BRLT delay		; if not matching , keep checking

RET			; if timer/counter is finally matching, return

- VERIFICATION USING C OF TASK 2

#define F_CPU 16000000UL //Change frequency to 16MHz

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<2);	//PB2 is an output, XXXX X1XX

PORTB |= (1<<2);	//PB2 set as high to turn off LED, XXXX X1XX

DDRC &= (0<<2);	//PC2 is an input, XXXX X0XX

PORTC |= (0<<2);	//PC2 set as low or unpressed, XXXX X0XX

while (1) {

if (!(PINC & (1 << PINC2))){ // If button is pressed

PORTB &= ~(1<<2); 	// turn on LED

_delay_ms(1250); 	// delay for 1250ms

}

else { 			// if button is not pressed

PORTB |= (1<<2); // set PB2 to high or LED off

}

}

}

- SCHEMATICS

Task 1:

Task 2:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

TASK 1: Initial State (Note that PORTB2 as 0 means LED is turned ON)

TASK 1: After .435 Sec (PORTB2 is 1 which means LED is OFF)

TASK 1: After .290 Sec

TASK 2: INITIAL BUTTON = 0 (or PRESSED)

TASK 2: AFTER DELAY

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

TASK 1:

TASK 2:

- VIDEO LINKS OF EACH DEMO

DA2A Task 1- https://youtu.be/6_uJ5-JgL44

DA2A Task 2 - https://youtu.be/mwjTFpltsVw

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ron Joshua Recrio