CPE301 – SPRING 2019

# Design Assignment 2C

Student Name: Ron Joshua Recrio

Student #: 5003825419

Student Email: recrio@unlv.nevada.edu

Primary Github address: https://github.com/recrio

Directory: /DesignAssignments/DA2C

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

ATmega328p Xplained Mini

Multifunction Shield

Block diagram with pins used in the Atmega328P

Task 1-3 for Task 1:

Task 1-3 for Task 2:

- INITIAL CODE OF DA2A

DA2A Task 1:

#include <stdio.h>

#include <avr/io.h>

int main(void)

{

DDRB = (1<<2); //Make PB2 Output

TCCR1B = 5;	// set prescaler to 1024

while (1)

{

TCNT1 = 0; // set timer/counter to 0

while (TCNT1 != 6796) {

//do nothing

}

PORTB ^= (1<<2); // toggle PB2 using xor

TCNT1 = 0; // reset again

while (TCNT1 != 4531) {

// do nothing

}

PORTB ^= (1<<2); // toggle PB2 using xor

}

}

DA2A Task 2:

#define F_CPU 16000000UL //Change frequency to 16MHz

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<2);	//PB2 is an output, XXXX X1XX

PORTB |= (1<<2);	//PB2 set as high to turn off LED, XXXX X1XX

DDRC &= (0<<2);	//PC2 is an input, XXXX X0XX

PORTC |= (0<<2);	//PC2 set as low or unpressed, XXXX X0XX

while (1) {

if (!(PINC & (1 << PINC2))){ // If button is pressed

PORTB &= ~(1<<2); 	// turn on LED

_delay_ms(1250); 	// delay for 1250ms

}

else { 			// if button is not pressed

PORTB |= (1<<2); // set PB2 to high or LED off

}

}

}

- DEVELOPED CODE OF TASK 1/C FROM DA2A

DA2C Task 1 for Task 1:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

int main(void)

{

DDRB = (1<<DDB2); // Make PB2 Output

PORTB = (0<<DDB2); // Turn on LED

TCCR0A = 0; // Normal Mode

TCCR0B = 5; // Set prescaler to 1024

int ovrflow = 0; // overflow counter

while (1)

{

TCNT0 = 0; // Reset counter

ovrflow = 0; // Reset overflow counter

// Delay for 6796

while (ovrflow < 26) { // Gets to 6656

while ((TIFR0 & 0x01) == 0) {}

ovrflow++; // increment ovrflow

TCNT0 = 0; // reset counter

TIFR0 = 1; // reset ovf flag

}

while (TCNT0 < 140) {} // 6656+140 = 6796

PORTB ^= (1<<DDB2); // Turn off LED

TCNT0 = 0;	// reset counter

ovrflow = 0; // reset ovrflow counter

// Delay for 4531

while (ovrflow < 17) { // 4352

while ((TIFR0 & 0x01) == 0) {}

ovrflow++; // increment ovrflow

TCNT0 = 0; // reset counter

TIFR0 = 1; // reset ovf flag

}

while (TCNT0 < 179) {} // 4352+179 = 4531

PORTB ^= (1<<DDB2); // Turn on LED

}

}

DA2C Task 1 for Task 2:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

int main(void)

{

DDRB = (1<<DDB2); // Make PB2 Output

DDRC = (0<<DDC2); // Make PC2 Input

PORTB = (1<<DDB2); // Turn off LED

PORTC = (1<<DDC2); // Turn on pull-up transistor

TCCR0A = 0; // Normal Mode

TCCR0B = 5; // Set prescaler to 1024

int ovrflow = 0; // overflow counter

while (1)

{

if (!(PINC & (1 << PINC2))) {	// if button pressed

PORTB ^= (1<<DDB2); // Turn on LED

TCNT0 = 0; // Reset counter

ovrflow = 0; // Reset overflow counter

// Delay for 1.25 sec (19531 TCNT)

while (ovrflow < 76) { // Gets to 19456

while ((TIFR0 & 0x01) == 0) {}

ovrflow++; // increment ovrflow

TCNT0 = 0; // reset counter

TIFR0 = 1; // reset ovf flag

}

while (TCNT0 < 75) {} // 19456+75 = 19531

PORTB ^= (1<<DDB2); // Turn off LED

}

}

}

- DEVELOPED CODE OF TASK 2/C FROM DA2A

DA2C Task 2 for Task 1:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

int ovrflow = 0; // global ovrflow counter

int main(void)

{

DDRB = (1<<DDB2); // Make PB2 Output

PORTB = (0<<DDB2); // Turn on LED

TIMSK0 |= (1<<TOIE0); // Set up interrupt

TCCR0A = 0; // Normal Mode

sei(); // interrupt enable

TCCR0B = 5; // Set prescaler to 1024

while (1)

{

}

}

ISR (TIMER0_OVF_vect) {

ovrflow++; //increment ovrflow

if (ovrflow == 26) { // delay for .435s

TCNT0 = 0;

while (TCNT0 < 140) {}

PORTB ^= (1<<DDB2); // Turn OFF

TCNT0 = 0; // reset counter

}

else if (ovrflow == 43) { // delay for .29s

TCNT0 = 0;

while (TCNT0 < 179) {}

PORTB ^= (1<<DDB2); // Turn ON

ovrflow = 0; // reset ovrflow

TCNT0 = 0; // reset counter

}

TCNT0 = 0;

}

DA2C Task 2  for Task 2:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

int ovrflow = 0; // global ovrflow counter

int main(void)

{

DDRB = (1<<DDB2); // Make PB2 Output

DDRC = (0<<DDC2); // Make PC2 Input

PORTB = (1<<DDB2); // Turn off LED

PORTC = (1<<DDC2); // Turn on pull-up transistor

TIMSK0 |= (1<<TOIE0); // Set up interrupt

TCCR0A = 0; // Normal Mode

sei();

TCCR0B = 5; // Set prescaler to 1024

while (1)

{

if (!(PINC & (1 << PINC2))) { // if button pressed

PORTB ^= (1 << DDB2); // Turn on LED

TCNT0 = 0;

ovrflow = 0;

while (!(PORTB & (1<<PORTB2))) {} // while on

}

ovrflow = 0;

}

}

ISR (TIMER0_OVF_vect) {

ovrflow++; //increment ovrflow

if (ovrflow == 76) {

TCNT0 = 0;

while (TCNT0 < 75) {}

PORTB ^= (1<<DDB2); // Turn off LED

}

TCNT0 = 0; //reset counter

}

- DEVELOPED CODE OF TASK 3/C FROM DA2A

DA2C Task 3 for Task 1:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

int ovrflow = 0; // global ovrflow counter

int main(void)

{

DDRB |= (1<<DDB2); // Make PB2 Output

PORTB &= (0<<DDB2); // Turn on LED

TIMSK0 |= (1<<OCIE0A); // Set up interrupt

TCCR0A |= (1<<WGM01); // Normal Mode

OCR0A = 0xFF;

TCNT0 = 0;

sei(); // interrupt enable

TCCR0B |= (1<<CS02) | (1<<CS00); // Set prescaler to 1024

while (1)

{

}

}

ISR (TIMER0_COMPA_vect) {

ovrflow++; //increment ovrflow

if (ovrflow == 26) { // delay for .435s

TCNT0 = 0;

while (TCNT0 < 140) {}

PORTB ^= (1<<DDB2); // Turn OFF

TCNT0 = 0; // reset counter

}

else if (ovrflow == 43) { // delay for .29s

TCNT0 = 0;

while (TCNT0 < 179) {}

PORTB ^= (1<<DDB2); // Turn ON

ovrflow = 0; // reset ovrflow

TCNT0 = 0; // reset counter

}

}

DA2C Task 3 for Task 2:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

int ovrflow = 0; // global ovrflow counter

int main(void)

{

DDRB |= (1<<DDB2); // Make PB2 Output

DDRC &= (0<<DDC2); // Make PC2 Input

PORTB |= (1<<DDB2); // Turn off LED

PORTC |= (1<<DDC2); // Turn on pull-up transistor

TIMSK0 |= (1<<OCIE0A); // Set up interrupt

TCCR0A |= (1<<WGM01); // Normal Mode

OCR0A = 0xFF;

TCNT0 = 0;

sei();

TCCR0B |= (1<<CS02) | (1<<CS00); // Set prescaler to 1024

while (1)

{

if (!(PINC & (1 << PINC2))) { // if button pressed

PORTB ^= (1 << DDB2); // Turn on LED

TCNT0 = 0;

ovrflow = 0;

while (!(PORTB & (1<<PORTB2))) {} // while on

}

ovrflow = 0;

}

}

ISR (TIMER0_COMPA_vect) {

ovrflow++; //increment ovrflow

if (ovrflow == 76) {

TCNT0 = 0;

while (TCNT0 < 75) {}

PORTB ^= (1<<DDB2); // Turn off LED

}

TCNT0 = 0; //reset counter

}

- SCHEMATICS

Task 1-3 for Task 1:

Task 1-3 for Task 2:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

DA2C Task 1_1 (BEFORE):

DA2C Task 1_1 (AFTER):

DA2C Task 1_1 Waveform:

DA2C Task 1_2 (BEFORE):

DA2C Task 1_2 (AFTER):

DA2C Task 1_2 Waveform:

DA2C Task 2_1 (BEFORE):

DA2C Task 2_1 (AFTER):

DA2C Task 2_1 Waveform:

DA2C Task 2_2 (BEFORE):

DA2C Task 2_2 (AFTER):

DA2C Task 2_2 Waveform:

DA2C Task 3_1 (BEFORE):

DA2C Task 3_1 (AFTER):

DA2C Task 3_1 Waveform:

DA2C Task 3_2 (BEFORE):

DA2C Task 3_2 (AFTER):

DA2C Task 3_2 Waveform:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1-3:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/hjMDncpwhuI

- GITHUB LINK OF THIS DA

https://github.com/recrio/submissions/tree/master/DesignAssignments/DA2C

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ron Joshua Recrio