CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Ron Joshua Recrio

Student #: 5003825419

Student Email: recrio@unlv.nevada.edu

Primary Github address: https://github.com/recrio/submissions

Directory: /DesignAssignments/DA4A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

ATmega328p Xplained Mini

Multifunction Shield

28BYJ-48 (Stepper Motor)

ULN2003A (Unipolar Stepper Motor Driver)

Block diagram with pins used in the Atmega328P

Task 1:

Task 2:

- INITIAL CODE OF TASK 1/B

#define F_CPU 16000000UL

#define CONVERSION 10

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void TIMER_init(void);

void ADC_init(void);

volatile int ADCvalue; // holds the value of ADC

int main(void)

{

DDRB = 0x0F; // PORTB[3:0] set to output

PORTB = 0; // Initially set output to 0

ADC_init(); // initializes ADC

TIMER_init(); // initializes TIMER

while (1)

{

ADCSRA |= (1<<ADSC); // Start conversion

while((ADCSRA&(1<<ADIF))==0); // while not done converting do nothing

ADCSRA |= (1<<ADIF); // reset converter

ADCvalue = ADC; // store ADC to ADCvalue for debugging

OCR1A = CONVERSION*ADCvalue; // 224/1024 is the ratio of the speed so that max speed is 95% of the pwm

// Sequence

PORTB = 0x09;

while(!(TIFR1 & (1<<OCF1A)));   // Delays using CTC

TIFR1 |= (1 << OCF1A);			// Resets

PORTB = 0x03;

while(!(TIFR1 & (1<<OCF1A)));

TIFR1 |= (1 << OCF1A);

PORTB = 0x06;

while(!(TIFR1 & (1<<OCF1A)));

TIFR1 |= (1 << OCF1A);

PORTB = 0x0C;

while(!(TIFR1 & (1<<OCF1A)));

TIFR1 |= (1 << OCF1A);

}

}

void TIMER_init(void) {

TCCR1B = (1<<WGM12) | // CTC mode

(1<<CS11); // Prescaler 8

}

void ADC_init(void) {

DDRC &= (0<<PORTC0); // PC0 as input

PORTC |= (1<<PORTC1); // Pull up resistor

ADMUX |= (1<<REFS0); // REFERENCE VOLTAGE AT AREF

ADCSRA |=

(1<<ADEN)| // ADC enable

(1<<ADPS2)| // ADC

(1<<ADPS1)| // Prescaler

(1<<ADPS0); // 128

sei();

}

- INITIAL CODE FOR TASK 2/B

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

void TIMER_init(void);

void ADC_init(void);

int main(void)

{

ADC_init(); // initialize ADC settings

TIMER_init(); // initialize TIMER1

while(1)

{

ADCSRA |= (1 << ADSC);	// Starts conversion

while((ADCSRA&(1<<ADIF))==0); // wait for conversion

ADCSRA |= (1<<ADIF); // reset converter

ICR1 = 4999; 	// fPWM = 50Hz

DDRB |= (1<<PB1);	// Set PB1 as output

OCR1A = ADC;	// Adjusts pulse width range

_delay_ms(100);	// Short Delay

}

}

void TIMER_init(void)

{

//Timer1

TCCR1A = (1<<COM1A1) | // Clear OC1A on Compare non-inverting

(1<<COM1B1) | //

(1<<WGM11)  ; // Fast

TCCR1B = (1<<WGM13) |  // PWM

(1<<WGM12) |  // TOP = ICR1

(1<<CS11) |  // Prescaler

(1<<CS10);	  // 64

}

void ADC_init (void)

{

ADMUX = (1<<REFS0);											// Set Aref

// right-justified data

ADCSRA |= (1<<ADEN) |	// ADC enable

(1<<ADSC) |	// AD start conversion

(1<<ADPS2)|	// ADC

(1<<ADPS1)|	// Prescaler

(1<<ADPS0);	// 128

}

- SCHEMATICS

Task 1:

*not exact models

Task 2:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task1:

Before

After

Task2:

Before

After

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1:

Task 2:

- VIDEO LINKS OF EACH DEMO

Task 1:

https://youtu.be/ouQdYaP1QoE

Task 2:

https://youtu.be/pdVJZY0RRas

- GITHUB LINK OF THIS DA

https://github.com/recrio/submissions/tree/master/DesignAssignments/DA4B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ron Joshua Recrio