CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Ron Joshua Recrio

Student #: 5003825419

Student Email: recrio@unlv.nevada.edu

Primary Github address: https://github.com/recrio/submissions

Directory: /DesignAssignments/DA5

Partner: Steven Lee

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

ATmega328p Xplained Mini

NRF24L01

LM35

Block diagram with pins used in the Atmega328P

- INITIAL CODE OF TASK 1/B

settings changed in lib file

main.c

//	Set clock frequency

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//	Set up UART for printf();

#ifndef BAUD

#define BAUD 9600

#endif

#include "inc\STDIO_UART.c"

//	Include nRF24L01+ library

#include "inc\nrf24l01.c"

#include "inc\nrf24l01-mnemonics.h"

#include "inc\spi.c"

void print_config(void);

//	Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

void ADC_init (void);

volatile unsigned adc_temp;

volatile char temp[20];

int main(void)

{

//	Set cliche message to send (message cannot exceed 32 characters)

char tx_message[32];				// Define string array

strcpy(tx_message,"It's working!");	// Copy string into array

//	Initialize UART

uart_init();

//  Initialize ADC

ADC_init();

//	Initialize nRF24L01+ and print configuration info

nrf24_init();

print_config();

//	Start listening to incoming messages

nrf24_start_listening();

strcpy(tx_message,"Initializing Chat Room...");	// Copy string into array

nrf24_send_message(tx_message);

while (1)

{

if (message_received)

{

//	Message received, print it

message_received = false;

printf("Received Temperature: %s\n\n",nrf24_read_message());

ADCSRA |= (1<<ADSC); // start conversion

while ((ADCSRA&(1<<ADIF))==0){} // Wait for conversion

ADCSRA |= (1<<ADIF); // Clear Interrupt Flag

adc_temp = ADCL; // take in lower bits first

adc_temp = adc_temp | (ADCH<<8); // take in upper bits

adc_temp = (adc_temp/1024.0) * 5000/10;

adc_temp = adc_temp*1.8 + 32;

snprintf(temp,sizeof(temp),"%d\r\n",adc_temp); // print to the buffer

//	Send message as response

_delay_ms(500);

status = nrf24_send_message(temp);

if (status == true) printf("Temperature Successfully Sent\n\n");

}

}

}

//	Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

//  Prints configuration

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

void ADC_init(void) {

ADMUX |= (1<<REFS0)| // Selected AVcc

(1<<MUX2) ;

ADCSRA |= (1<<ADEN)| // Enable ADC

(0<<ADSC)| // Do not start conversion

(0<<ADATE)|// Auto Trigger Disabled

(0<<ADIF)| // Interrupt Flag Cleared

(0<<ADIE)| // Interrupt Disabled

(1<<ADPS2)| // ADC Prescaler Select

(0<<ADPS1)| // Set to

(1<<ADPS0); // 32

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/ZMXrQmhXJXM

- GITHUB LINK OF THIS DA

https://github.com/recrio/submissions/tree/master/DesignAssignments/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ron Joshua Recrio